FURTHER EDUCATION AND TRAINING CERTIFICATE: INFORMATION TECHNOLOGY: SYSTEMS DEVELOPMENT ID 78965 LEVEL 4 – CREDITS 163.

LEARNER GUIDE SAQA: 114636 DEMONSTRATE AN UNDERSTANDING OF PREVENTATIVE MAINTENANCE, ENVIRONMENTAL AND SAFETY ISSUES IN A COMPUTER ENVIRONMENT

Learner Information:

| Details | Please Complete this Section |
| --- | --- |
| Name & Surname: |  |
| Organisation: |  |
| Unit/Dept: |  |
| Facilitator Name: |  |
| Date Started: |  |
| Date of Completion: |  |

Copyright

All rights reserved. The copyright of this document, its previous editions and any annexures thereto, is protected and expressly reserved. No part of this document may be reproduced, stored in a retrievable system, or transmitted, in any form or by any means, electronic, mechanical, photocopying, recording or otherwise without the prior permission.

Key to Icons

The following icons may be used in this Learner Guide to indicate specific functions:

| Books | This icon means that other books are available for further information on a particular topic/subject. |
| --- | --- |
| References | This icon refers to any examples, handouts, checklists, etc… |
| Important | This icon represents important information related to a specific topic or section of the guide. |
| Activities | This icon helps you to be prepared for the learning to follow or assist you to demonstrate understanding of module content. Shows transference of knowledge and skill. |
| Exercises | This icon represents any exercise to be completed on a specific topic at home by you or in a group. |
| Tasks/Projects | An important aspect of the assessment process is proof of competence. This can be achieved by observation or a portfolio of evidence should be submitted in this regard. |
| Workplace Activities | An important aspect of learning is through workplace experience. Activities with this icon can only be completed once a learner is in the workplace |
| Tips | This icon indicates practical tips you can adopt in the future. |
| Notes | This icon represents important notes you must remember as part of the learning process. |

Learner Guide Introduction

| About the Learner Guide… | This Learner Guide provides a comprehensive overview of the DEMONSTRATE AN UNDERSTANDING OF PREVENTATIVE MAINTENANCE, ENVIRONMENTAL AND SAFETY ISSUES IN A COMPUTER ENVIRONMENT , and forms part of a series of Learner Guides that have been developed for FURTHER EDUCATION AND TRAINING CERTIFICATE: INFORMATION TECHNOLOGY: TECHNICAL SUPPORT ID 78964 LEVEL 4 – 163 CREDITS. The series of Learner Guides are conceptualized in modular’s format and developed for NATIONAL CERTIFICATE: MEASUREMENT, CONTROL AND INSTRUMENTATION. They are designed to improve the skills and knowledge of learners, and thus enabling them to effectively and efficiently complete specific tasks. Learners are required to attend training workshops as a group or as specified by their organization. These workshops are presented in modules, and conducted by a qualified facilitator. |
| --- | --- |
| Purpose | The purpose of this Learner Guide is to provide learners with the necessary knowledge related to DEMONSTRATE AN UNDERSTANDING OF PREVENTATIVE MAINTENANCE, ENVIRONMENTAL AND SAFETY ISSUES IN A COMPUTER ENVIRONMENT. |
| Outcomes | People credited with this unit standard are able to: - Demonstrate an understanding of the use of preventive maintenance measures and procedures of computer hardware; - Demonstrate an understanding of the use of safety measures and procedures of a computer working environment; - Explain environmental protection measures and procedures of a computer working environment. |
| Assessment Criteria | The only way to establish whether a learner is competent and has accomplished the specific outcomes is through an assessment process. Assessment involves collecting and interpreting evidence about the learner’s ability to perform a task. This guide may include assessments in the form of activities, assignments, tasks or projects, as well as workplace practical tasks. Learners are required to perform tasks on the job to collect enough and appropriate evidence for their portfolio of evidence, proof signed by their supervisor that the tasks were performed successfully. |
| To qualify | To qualify and receive credits towards the learning programme, a registered assessor will conduct an evaluation and assessment of the learner’s portfolio of evidence and competency |
| Range of Learning | This describes the situation and circumstance in which competence must be demonstrated and the parameters in which learners operate |
| Responsibility | The responsibility of learning rest with the learner, so: - Be proactive and ask questions, - Seek assistance and help from your facilitators, if required. |

Learning Unit1

Demonstrate an understanding of preventative maintenance, environmental and safety issues in a computer environment

UNIT STANDARD NUMBER	: 	114636

LEVEL ON THE NQF		: 	3

CREDITS			: 	6

FIELD				: 	Physical, Mathematical, Computer and Life Sciences

SUB FIELD			: 	Information Technology and Computer Sciences

| PURPOSE: | This unit standard is designed to provide credits towards health and safety in the work environment, specifically applicable to the computer industry. Qualifying learners are able to: - Demonstrate an understanding of the use of preventive maintenance measures and procedures of computer hardware; - Demonstrate an understanding of the use of safety measures and procedures of a computer working environment; - Explain environmental protection measures and procedures of a computer working environment. The performance of all elements is to a standard that allows for further learning in this area. |
| --- | --- |
| LEARNING ASSUMED TO BE IN PLACE: |  |
| - The credit value is based on the assumption that people starting to learn towards this unit standard are:  Competent in Communications Literacy at NQF level 2 - Able to describe basic concepts of Electrostatic Discharge (ESD) - Able to describe basic concepts of electricity -  Able to identify basic computer components |  |

SESSION 1 Demonstrate an understanding of the use of preventive maintenance measures and procedures.

Learning Outcomes

- The demonstration identifies the causes of computer hardware maintenance. - The demonstration covers preventive measure for the causes of computer hardware maintenance.

Demonstrate an understanding of the use of preventive maintenance measures and procedures.

Preventive Maintenance Products and Procedures

Preventive maintenance is something that customers often forget or do not believe is necessary. However, nothing could be further from the truth. For example, suppose you have a customer who frequently reschedules their preventive maintenance time. Because you are unable to perform the maintenance, neither of you would be aware that the fan in the power supply had stopped working. As a result, instead of just having to replace the power supply, the processor and some of the memory failed due to the excessive heat. In addition, the unscheduled downtime resulted in a loss of productivity and a larger bill from you.

As you can see from the preceding example, by performing regular maintenance on computer equipment, you can extend the life of the components themselves as well as locating potential problems. In the long run, this will save the customer time and money. In the next subsections, we discuss the various products available on the market along with the proper procedures for using them.

Manufacturer Suggested Guidelines

Before beginning any preventative maintenance procedures, it is critical that you consult the manufacturer’s documentation. Vendors include the information on the proper cleaning materials to use when cleaning or maintaining their components. Failure to follow these guidelines could result in either component degradation, complete failure, or a voided warranty. Never assume that you already know what you can use with what device; instead, take the time to review the documentation. Remember that customer service is not only solving a problem, but also ensuring that you do not generate them.

Liquid Cleaning Compounds

There are several liquid cleaning compounds that are used when you perform preventive maintenance. However, it is important to keep in mind that you should always refer to the manufacturer’s documentation prior to using any cleaning compound on a component. This is because every vendor may use different materials in the component itself. In addition, some vendors require specialized cleaning compounds that can be purchased from them.

Various forms of alcohol are frequently used in cleaning computer components, such as isopropyl alcohol and denatured alcohol. These items are generally used to clean contacts and are applied to special disks used for cleaning floppy drive read/write heads. Mild detergent can be used on the outside of the monitor, the computer case, and on keyboards.

Cleaning Contacts and Connectors

Few people realize, or remember, that contacts and connectors require cleaning. The reason people forget is because connectors are seldom removed from their respective sockets. Nevertheless, these items do get dirty over time and do require cleaning. Most components can be cleaned with a cotton swab that has been coated with isopropyl alcohol. However, many manufacturers recommend that you use a pencil eraser to clean the contacts on expansion cards. As with most computer components, it is important that you consult the vendor’s documentation to ensure that their product will not have an adverse effect from the materials that you use.

Cleaning Tools

There are several cleaning tools that you should keep in your maintenance kit. One nice item to have is a rubber knife. When you are called upon to remove hardened residue from metal components, you can use a rubber knife to dislodge particles that the vacuum or dust-free cloth could not remove. Never use a metal knife or other metallic object when cleaning the computer or its components, as you can either damage them or cause injury to yourself through electrostatic discharge (ESD). ESD occurs when two charged objects come into contact with each other, such as your hand against a door knob. The charge is transferred between the two objects until both objects have an equal charge.

Compressed air comes in handy when you are cleaning components, such as a keyboard, or areas that a vacuum can not get to (or shouldn’t get to!). Compressed air is distributed in an aerosol-type can and is available at any computer or electronics store. The air is compressed in the can, hence its name, by extreme pressure and dispelled through a nozzle that is similar to the old aerosol cans. You would use compressed air when you need to blow dust, dirt, or other unwanted debris out and away from a component.

Dust-free, lint-free disposable cloths or wipes are also cheaply available and should be used whenever you need to wipe the surface of a component. Normal cloths naturally attract lint and dust, while dust-free or lint-free cloths do not. This helps to ensure that you are not contributing harmful materials to sensitive components.

Floppy Drive and Tape Head Cleaning

When you’ve determined that the read/write heads on a floppy drive need cleaning, you need to obtain a floppy drive cleaning kit. The kits contain a special cleaning disk that looks like a normal floppy and a small bottle of isopropyl alcohol. The disk has an access hole, similar to a regular disk, on which you place a few drops of the isopropyl alcohol. Then you insert the disk into the drive and the drive will spin up. When the read/write heads attempt to read it, the disk will spin over the heads and clean them. These kits are relatively inexpensive and can be found at most computer and electronics stores.

When you clean a tape drive, you are performing the same function as with a floppy drive. The only difference is that you would use a special tape-cleaning cartridge instead of a cleaning disk with isopropyl alcohol. The cleaning cartridge looks exactly like a normal tape cartridge, although some manufacturers use a different color (usually white or beige) casing around a cleaning cartridge to help differentiate it from a normal cartridge.

Hard Drive Maintenance

Because hard drives are sealed units that can only be opened in a "clean room," there is nothing you can do to clean the read/write heads. However, hard drives have a tendency to become fragmented over a period of time or with excessive use and should be defragmented as a part of normal preventative maintenance. To perform a defragmentation, you can use a special software program that was designed specifically for this purpose, such as Norton Utilities or Microsoft Disk Defragmenter.

Another problem with hard drives is that the disk surface can become corrupted over time. If the drive attempts to read or write to these corrupted sections, the computer could crash or exhibit strange symptoms that are difficult to pin down. Some software utilities, such as the Microsoft Scandisk utility, are designed to locate and mark corrupted sections on the disk. The mark tells the hard drive to ignore this section of the hard drive. This process should be performed on a regular basis in conjunction with a disk defragmentation.

Determining Wear and Tear

Computer peripherals are not only electrical components, they are also mechanical components. These components wear out over time and eventually fail. As a service technician, you should inspect peripherals for signs of deterioration and replace any components that are about to expire. By doing so, you will save the customer time and money from untimely downtime when the component finally fails. Signs of wear can include thin spots on belts, bends or tear in cables, and moving parts that are only sporadically functioning.

Vacuuming

Most people do not realize it, but it is important to vacuum the inside of a computer case whenever you open it. Dust and dirt particles get sucked into the case through the air ducts and deposit themselves anywhere they can. These particles can conduct an electrical charge, resulting in possible damage to the delicate electronic components inside. Most offices do not have a vacuum that you will be able to use, as there are usually cleaning crews that carry their own equipment with them. However, small, portable vacuums are available for as little as $20 and are well worth the investment.

Whenever you have to open a printer for repair, it is important that you vacuum out the interior. Bits of paper and dust have a tendency to accumulate on the inside of a printer at a rate that exceeds the interior of a computer. The reason that this happens is due to the nature of the printer; that is, producing output on paper medium. Small bits of paper can tear off and become lodged inside the printer from clearing a paper jam. The dust produced during the printing process itself also contributes to the mess. As stated earlier, dirt and dust can carry an electrical charge. This small charge can be enough to damage the electrical components inside of a computer, as well as causing excess wear and tear on mechanical components that will have to work a bit harder against the dirt.

Environmental Hazard Protection

Computers must have a reliable source of power in order to function. However, there are many problems with our power supply that have the potential to damage computer systems and their individual components. The uninterruptible power supply (UPS) is a device that was designed to protect your computer and its components from possible injury from the problems that are inherent with today’s existing power supply structure.

Power Issues

With society’s increasing demand for power comes a price. The power utility companies have been hard-pressed to keep up with the present demand, leading to problems in dealing with future increases. As a result, brownouts, which are the momentary lapses in power supply, have become more common than they used to. Brownouts can cause problems with computer components that are not designed to withstand these events. Blackouts are similar to brownouts, as they are also lapses in power, but they are long-term power outages.

When there is a power spike, there is a sudden, huge increase in power that lasts for a split second. Power spikes can literally fry computer components. A power surge is similar to a spike, except that a power surge may not have the intensity that a spike can have.

As stated previously, noise creeps into a power line and is transmitted to the computer. This noise is almost impossible to keep out of the power line due to the vast distances involved and the technology used to transmit power. Noise is a one of the most prevalent problems with the power supply today.

UPS (Uninterruptible Power Supply)

Today’s UPS units are populating homes and businesses at an increasing rate as the cost has decreased substantially from the early days of computer history. The UPS is designed to protect your computer from sudden lapses in power, power spikes or surges, and "dirty" current. This is accomplished by several components in the UPS, such as suppressors, noise filters, and surge protectors. Each of these items are discussed in the following paragraphs.

Suppressors

At times, your power outlet will experience momentary surges of current, called spikes. Spikes can harm computers and their components much in the same way as ESD damage. However, the damage that results is usually on a much greater level, with catastrophic damage becoming more frequent. A suppressor is designed to either absorb or block the excess power and thus save computer components from injury.

Noise Filters

When someone refers to "dirty current," they are talking about the noise present on the power line itself. This noise is caused by electro-magnetic interferance (EMI) and can stray, or leak, from the current into nearby components. When EMI leaks from power current, it is called a magnetic field and can easily damage computer components. UPS’s contain a special filter, called a noise filter, that reduces the amount of noise present in electrical current and eliminates magnetic fields caused by noise, thus providing some protection to the components that utilize the current or are nearby.

Computer Preventive Maintenance Procedures

Like any piece of machinery, a computer will last longer with routine maintenance. Over time, computers become cluttered with unneeded data while cases can fill up with dust. Properly maintaining your computer can keep it running fast and reduce the chance of hardware failure.

Hard Drive Maintenance

- The hard drive is one of the most important devices to maintain on a computer, because it contains the operating system and long-term storage space for data. All your programs, music, videos, documents and other files are stored on the hard drive. One important maintenance procedure is to run the disk defragmenter on your hard drives about once a month. A disk defragmenter attempts to realign data so that information dealing with a certain file or program is grouped together, which can improve loading times. Another important part of maintaining a hard drive is deleting old data that you don't need, and backing up files that you want to save on a storage drive. Allowing a hard drive to become full of old programs and files will increase the rate that it becomes fragmented, and leave less room for new data.

Internet Security

- Another important aspect of computer maintenance is preventing and getting rid of viruses and other malware on the Internet. You should always run a fully updated antivirus program when you are using the Internet, and you should perform full system scans with your antivirus program at least once a week. Since viruses can sometimes get past antivirus programs, it is a good idea to run a firewall to block potential threats. In addition to these preventative measures, extra anti spyware and malware programs can be used to scan the computer as an additional layer of security. Spybot Search and Destroy and Lavasoft's Ad Adware are two popular malware removal tools.

Case Cleaning

- Even if your hard drive is squeaky clean and free of viruses, your computer's physical case can be filthy. Cases usually contain several fans that move air around, which causes dust to flow through your computer that can build up over time. To keep case fans and other devices from being hampered by dust accumulation, open your case and get rid of dust every few months. To do this, use a screwdriver to remove the left side panel of your computer case, then use canned air to blow the dust out of your computer. Allowing dust to build up can cause your computer to run hotter, and heat is one of the primary reasons for computer crashes and hardware failure.

Preventive Maintenance Procedures

If they are carried out on a regular basis, these regularly scheduled Windows PC procedures will keep a system from getting so "out-of-tune" that a system crash is imminent, and will also prevent other problems before they occur. Regular maintenance in Windows system covers basics such as cleaning up, backing-up data, removing obsolete programs, Microsoft updates, other security updates and defragmentation.

Cleaning Up

Every computer builds up temporary material. Files are generated and the system registry will add more entries. A computer user needs to regularly clean the file system and registry.

Back-Up Data

If the hard drive fails, any data may be lost completely. So it should be backed up regularly to an eternal storage resource like CD, DVD or external hard drive.

Remove Obsolete Programs

A computer owner may have installed many programs. It is good practice to uninstall obsolete programs, since they not only take up valuable space, but they will also use system resources.

Microsoft Updates

- Microsoft updates must be applied since they will enhance the system and also prevent problems occurring later on, and these also help protect against harmful threats applied within documents.

Virus and Malware Checker Updates

- It is important to keep anti-virus and anti-malware programs updated. Many of these database updates are automatic. The owner is unprotected until the program is updated, and this is when the user is vulnerable to threats.

Defragmentation

Windows PCs require regular defragmentation. This should be done weekly and particularly if there is a client email program like Microsoft Outlook because it will write to the drive often.

The demonstration identifies the causes of computer hardware maintenance.

What Causes My Computer to Heat Up?

Overheating can have several consequences for you and your computer—some are minor, others are more serious. For example, with laptop computers, overheating can simply be uncomfortable for you when you place the computer on your lap. But beyond this type of inconvenience, overheating can have lasting negative effects on your computer including a failed logic board and continual freezing.

Fan Malfunction

- Overheating can be caused by issues with the computer's fan. All computers have fans that are triggered when the CPU reaches a certain temperature. However, these fans can malfunction for several reasons. One major cause of fan problems is simple overuse. The fans on your computer, like any other electronic device, are subject to wear and tear. Furthermore, fans can become clogged with dust and other small particles, causing them to run slowly or stop completely. When this happens, the fan cannot cool the internal components of the computer properly and the computer overheats.

Overworking the CPU

- When your computer's CPU is overtaxed, your computer can also overheat. This may occur when you have multiple applications running simultaneously. When the CPU's resources are allocated to many applications, your computer may overwork the CPU. Specifically, graphics-heavy applications like games or video editing software can cause your CPU to become overburdened, which in turn, overheats your computer.

Environment

- The environment where you use your computer can also contribute to overheating. It's a simple principle, but one that not every computer user may be familiar with: if your computer is placed in direct sunlight or close to a heat source such as a radiator or heat vent, the likelihood of your computer overheating increases.

Uneven Surfaces

- Another cause of overheating in laptops is use on uneven or soft surfaces. Laptops are designed to be placed on a flat surface, allowing the heat to escape from the back or sides of the machine. When a laptop is placed on an uneven surface, heat may not be able to escape properly. Compounding the problem, fans that are meant to cool your laptop may be blocked if the laptop is not used on an even surface.

Dust

Dust gives humans allergies, and it can cause the same problem with computer hardware. It can clog and accumulate in circuits, slots, contacts, ventilation ducts and fans. If not taken care of, this dust solidifies and prevents the optimal functioning of the system.  Therefore use an industrial strength dust blower to clean the inside of your casing at-least twice a month.

Rust

Rust is the universal killer of metallic objects. In some climates it can severely compromise your computer hardware maintenance. Rust is quick to catch onto the copper contacts of circuits, cables and the motherboard which can hinder the flow of current. It is important to keep your computer hardware in a neutral environment and away from direct heat, sunlight or cool, moist air.

Cooling

Heat is one of the main causes for accelerating the depreciation of the hardware. Heat causes greater wear and tear on the circuits and metal points. For proper computer hardware maintenance it is important you place the casing in an open area where there is plenty of air flow. You may have to install extra cooling fans to be on the safe side.

The demonstration covers preventive measure for the causes of computer hardware maintenance.

An uninterruptible power supply, also uninterruptible power source, UPS or battery/flywheel backup, is an electrical apparatus that provides emergency power to a load when the input power source, typically mains power, fails. A UPS differs from an auxiliary or emergency power system or standby generator in that it will provide near-instantaneous protection from input power interruptions, by supplying energy stored in batteries or a flywheel. The on-battery runtime of most uninterruptible power sources is relatively short (only a few minutes) but sufficient to start a standby power source or properly shut down the protected equipment.

A UPS is typically used to protect hardware such as computers, data centers, telecommunication equipment or other electrical equipment where an unexpected power disruption could cause injuries, fatalities, serious business disruption or data loss.

The primary role of any UPS is to provide short-term power when the input power source fails. However, most UPS units are also capable in varying degrees of correcting common utility power problems:

- Voltage spike or sustained Overvoltage

- Momentary or sustained reduction in input voltage.

- Noise, defined as a high frequency transient or oscillation, usually injected into the line by nearby equipment.

- Instability of the mains frequency.

- Harmonic distortion: defined as a departure from the ideal sinusoidal waveform expected on the line.

A small free-standing UPS. The unit in the photo has an IEC 60320 C14 input and three C13 outputs

Prevention and maintaining methods

We recommend using lightning protective electrical outlet in combination with a UPS .The UPS will assure quality electrical power and will prevent loss of data during a power outage. It will also provide you enough time to save any files you were working on. Another advantage of the UPS is that it protects your computer from shutting down improperly and it will take in any electrical socks. Electrical socks usually damage your system components, mainly the ones who function with electrical motors like hard drive, cd-rom, floppy disk.   For laptops, data lost can be avoided during a blackout if the battery is in. An Uninterruptible Power Supply (UPS), also known as an Uninterruptible Power Source, Continuous Power Supply (CPS) or a battery backup is a device which maintains a continuous supply of electric power to connected equipment by supplying power from a separate source when utility power is not available. There are two distinct types of UPS: off-line and line-interactive (also called on-line). The most commonly used is the line-interactive systems.

The on-line type of UPS, in addition to providing protection against complete failure of the utility supply, provides protection against all common power problems, and for this reason it is also known as a power conditioner.

SESSION 2 Demonstrate an understanding of the use of safety measures and procedures.

Learning Outcomes

- The demonstration explains safety measure for different types of fires and which applies to computer environments. - The demonstration covers potential hazards and safety procedures relating to the computer environment. - The demonstration cover Electrostatic Discharge (ESD) and precautions for it.

The use of safety measures and procedures.

Safety Precautions

Observe safety rules when working inside the system and when handling computer components. Avoid electric shock or personal injury by observing the following warning.

Hazardous Voltage

WARNING: before removing the system unit cover, turn off the power and unplug the system power cable. Power is removed only when the power cable is unplugged.

Electrostatic Discharge

WARNING: your computer's internal components are highly susceptible to risk of damage from electrostatic discharge (static electricity, which occurs naturally on your body due to friction). To avoid damaging your equipment, always earth yourself to your computer system before opening the computer's chassis or handling any internal component(s).

To do this, use the following procedure:

- For best results, we strongly recommend you purchase an anti-static wrist strap (available at electronic supply stores). To use it, connect the strap's metal “alligator clip” to an unpainted metal portion of your computer's chassis.

- Then connect the elastic wrist band to the strap, and slide the wrist band onto your wrist.

Safety Rules

- Select a suitable work space

- Avoid carpets in cool, dry areas.

- When you work on a carpet where static electricity is likely to be generated, make sure take anti-static measures beforehand.

Cloth

- Do not wear a wool or synthetic cloth to work with the computer.

- Wear anti-static shoes to work with the computer.

- Take off any jewels (a ring, bracelet, or wrist watch) before working with the computer.

Handling of components

- Keep any component in an anti-static bag until you actually install it to the computer.

- Hold an component by its edge to avoid touching any terminals or parts.

- To store or carry any component, place it in an anti-static bag.

- Turn off and disconnect all peripherals.

- Do not disassemble parts other than those specified in the procedure.

- Label any cable connectors before disconnecting. Note where the connector goes and in what position it was installed.

- Take care when connecting or disconnecting cables. A damaged cable can cause a short in the electrical circuit.

- When installing a cable, route the cable so it is not pinched by other components and is out of the path of the system unit cover.

- Prevent damage to the connectors by aligning connector pins before you connect the cable. Misaligned connector pins can cause damage to system components at power-on.

- When disconnecting a cable, always pull on the cable connector or strain-relief loop, not on the cable itself.

Working at our computers, most of us are not concerned about how our workstation is set up. We are more concerned about turning on the machine and starting to work. We then sit hunched over at the computer for a couple of hours and later wonder why we feel so tired at the end of the day. Our eyes may be blurred, our wrists may feel sore, our shoulders, necks and backs may ache a bit, and we feel totally drained.

Before starting to work at your computer, adjust the height, seat pan, seat tilt and back of your chair to your comfort.

Aside from just feeling fatigued, some computer users have experienced work-related injuries to the wrist, neck, shoulders or arms–sometimes permanently, partially as a result of improper workstation setup, bad work habits and just not knowing what precautions to take. Here are a few guidelines on setting up your workstation at home, school and in the office, as well as safe work habits to practice when using your computer:

- Before starting to work at your computer, adjust the height, seat pan, seat tilt and back of your chair to your comfort.

- The ideal chair is one where components are adjustable and your body is fully supported by the chair.

- Your feet should be flat on the floor and slightly in front of you. Knees should be at a comfortable open angle, thighs parallel to the floor, chair back slightly reclined and your back should be against the lumbar support. If your chair does not have a lumbar support, you should get a cushion that helps support your back

- Avoid crossing your legs or hugging the chair with your legs while using the computer. This inhibits blood flow, causing lower limb circulation problems, and increasing fatigue.

- You should be able to sit back in your chair comfortably and use the keyboard and mouse. Bring them to you! You should not be hunched over with your neck craned and your arms stretched out to use the mouse or keyboard. Try to relax your body as you work.

- Make sure all work tools (phone, often-used books) are within reach so that you do not have to do too much reaching or stretching.

You should not be hunched over with your neck craned and your arms stretched out to use the mouse or keyboard. Try to relax your body as you work.

- Ideally, your computer monitor and keyboard should be placed directly in front of you so that there is no twisting of your body as you type or use the mouse. In addition, placing the monitor at or slightly below eye level prevents the tilting up or down of your neck as you view the monitor.

- Your computer monitor should be positioned perpendicular to the light source in your work area so that there is no light shining directly on the monitor creating glare and reflections.

- Your wrists should be in a wrist-neutral position so that there is no bending, flexing or extension as you type. A neutral wrist is when your wrist is in one straight plane with your forearm.

- Hold on to your mouse, cup it gently with your hand and keep your wrist straight as you move it with your whole arm and not your wrist only.

- Change your body position frequently while maintaining good posture. A static posture over a long period of time will lead to fatigue and aches.

- Take regular stand and stretch breaks.

- Vary tasks to minimize repetitive activities. Prevent eye strain by looking away often and blinking.

Take regular stand and stretch breaks.

As you make these adjustments, always be aware of what your body is doing and how it feels. When you do this, you are actually practicing ergonomics, the science of how we interact with our physical work environment. The goal of ergonomics is physical comfort, fitting the task and work area to you, NOT adjusting your body to do the task. State-of-the-art furniture and accessories mean nothing if you do not adjust them to your body’s comfort.

Buyer beware! When shopping for furniture and accessories, look for physical comfort and listen to your body. The use of the word “ergonomic” in describing a product does not necessarily mean that it works or will work well for you.

The demonstration explains safety measure for different types of fires and which applies to computer environments.

Understand the Types of Fires that are Encountered

The National Fire Protection Association publishes a list of several different fire classifications. The three types of fires to be potentially found inside the modern data or computing center are:

Type A: Fires with combustible materials as its source, such as wood, cloth, paper, rubber, and many plastics

Type B: Fires in flammable liquids, oils, greases, tars, oil-base paints, lacquers, and flammable gases

Type C: Fires that involve electrical equipment

Choosing the Correct Fire Suppressant

Choose the correct type of fire suppressant for your computing or data room application. A fire suppressant that is capable of suppressing A class, B class and C class fires is preferable.

Many years ago, the use of a water-based fire sprinkler system was considered state-of-the-art for fire suppression design. A generation ago, with the advent of halon, the use of fire sprinkler systems began to be supplemented with fire fighting gaseous systems. Halon systems were controlled by electronic systems using smoke sensing technologies.

Fire Safety

Fire Safety Products

Fire Extinguishers

Fire Exit Signs

If a fire starts in the office environment, the usual policy is to get everyone out as quickly as possible and call the fire brigade.

However there may be times when a fire is discovered at an early stage and is considered controllable. In such a situation an attempt to extinguish the fire using fire extinguishing equipment should only be made if:

- Sufficient training has been provided on using fire extinguishers

- An escape route has been established

However fires can spread very quickly and if the situation escalates and becomes uncontrollable then it must be abandoned immediately.

Keep in mind the following safety precautions if attempting to extinguish a fire:

- Be sure you have the correct fire extinguisher for the type of fire and know how to use it.

- In commercial kitchens with built-in fire protection systems, the system should be activated before attempting to use a portable fire extinguisher.

- Maintain your exit. Leave yourself a way out in case the fire gets out of control.

- Have a back-up person standing by whenever possible in case assistance is needed.

- Stop and leave the area immediately if you experience any physical problems such as dizziness or difficulty breathing.

- Even if you think a fire is out, call the fire department to inspect the area. A fire may reignite or may still be smoldering where it cannot be seen.

- If a fire extinguisher has been used and is rechargeable, have it serviced before returning it to original location.

The demonstration covers potential hazards and safety procedures relating to the computer environment.

Electrical Hazards and Precautions

The use of any item of electrical equipment contains a risk element due to the potentially dangerous threat of electric shock. Computer systems and associated hardware, such as printers, scanners, network hubs and other devices, are commonly mains powered and as such are a potential hazard. The risk of electric shock applies to normal users of the equipment and also to those undertaking repairs or upgrades. For example, a fault could occur where the case of a PC becomes live and this could pose a risk of electrical shock to a user who inadvertently touches the case. Similarly when repairing a PC the technician could be at risk if they do not take the appropriate safety measures such as isolating the PC by disconnecting it from the mains supply.

In addition to the user observing good practice and minimising the risk of electric shock, there are other measures which can be taken to reduce the risk of injury to the operator.

In addition to minimising the personal risk we are also concerned with reducing the risk of fire and damage to other equipment. An electrical fault can generate large amounts of heat, which could pose a fire risk. An electrical fault in one piece of equipment could also damage other equipment and cables, etc.

The display screen

This should:

- display well-defined characters of adequate size and spacing

- have a stable image

- have easily adjustable brightness and contrast

- tilt and swivel easily to suit the user

- be free from glare and reflections

- use a separate base for the screen, or an adjustable table.

Now the most important computer hardware safety tip is to do with power and electrics.  The reason for this is that you can cause harm to yourself and to your computer if you are unaware of these safety issues.

Before opening your computer case and changing or upgrading parts, follow these steps:

- Fully shut down your computer

- Switch the power off button (at the back of the PC, if you have one)

- Turn off the power point and remove the power cable

- Remove other cables (eg. printers, monitors etc.)

- Wait a few minutes before proceeding to open the computer because power can still be running in the PC even after the power cable has been removed.

- Discharge static electricity - This is a crucial step.  It is important that you touch your hands to a non-electrical metal surface before touching any other components inside the computer, after opening the case off the PC.  This will prevent a static shock from frying the computer circuits.

The demonstration covers Electrostatic Discharge (ESD) and precautions for it.

Electrostatic discharge (ESD) is the sudden flow of electricity between two objects caused by contact, an electrical short, or dielectric breakdown. ESD can be caused by a buildup of static electricity by tribocharging, or by electrostatic induction.

ESD includes spectacular electric sparks, but also less dramatic forms which may be neither seen nor heard, yet still be large enough to cause damage to sensitive electronic devices. Electric sparks require a field strength above about 4 kV/cm in air, as notably occurs in lightning strikes. Other forms of ESD include corona discharge from sharp electrodes and brush discharge from blunt electrodes.

ESD can cause a range of harmful effects of importance in industry, including methane gas and coal dust explosions, as well as failure of solid state electronics components such as integrated circuits. These can suffer permanent damage when subjected to high voltages. Electronics manufacturers therefore establish electrostatic protective areas free of static, using measures to prevent charging such as avoiding highly charging materials, and measures to remove static such as grounding human workers, providing antistatic devices, and controlling humidity.

ESD simulators may be used to test electronic devices, for example with a human body model or a charged device model.

Causes of ESD

One of the causes of ESD events is static electricity. Static electricity is often generated through tribocharging, the separation of electric charges that occurs when two materials are brought into contact and then separated. Examples of tribocharging include walking on a rug, rubbing a plastic comb against dry hair, rubbing a balloon against a sweater, ascending from a fabric car seat, or removing some types of plastic packaging. In all these cases, the friction between two materials results in tribocharging, thus creating a difference of electrical potential that can lead to an ESD event.

Another cause of ESD damage is through electrostatic induction. This occurs when an electrically charged object is placed near a conductive object isolated from ground. The presence of the charged object creates an electrostatic field that causes electrical charges on the surface of the other object to redistribute. Even though the net electrostatic charge of the object has not changed, it now has regions of excess positive and negative charges. An ESD event may occur when the object comes into contact with a conductive path. For example, charged regions on the surfaces of styrofoam cups or bags can induce potential on nearby ESD sensitive components via electrostatic induction and an ESD event may occur if the component is touched with a metallic tool.

How to prevent ESD

ESD is avoided by preventing the build-up of static electricity. An ESDS and bodies within proximity of about one meter of the ESDS should be grounded so that electrostatic charge is bled off. If it is not possible for the ESDS and surrounding bodies to be grounded, then the ESDS should be shielded within conductive package, thus preventing the build-up of electrical charge around the ESDS, and any ESD is absorbed by the packaging.

To prevent ESD damage observe the following precautions:

- Wear an ESD grounding strap connected to a proper ground when handling modules.

- Keep boards in their antistatic protection bags until you are ready to install them.

- Do not touch pins, leads, or solder connections on the boards.

- Handle modules only by their edges, ejector handles, or stiffener rails.

- Store and ship modules in antistatic protection packaging.

Wrist strap

Each employee should have their own wrist strap. The wrist strap is the primary mean of controlling static charge on personnel. When the wrist strap is connected to ground, it keeps the employee near ground potential. Because the employee and other grounded objects in the workstation are at the same potential, there can be no hazardous discharge between them. Static charges are safely dissipated from the person to ground and do not accumulate.

The wrist strap must be worn against the skin. The employee must not be connected directly to the ground, but must be connected through a 1MΩ resistor. This is a safety requirement to prevent electrical shock if the employee comes into contact with high voltage while at the ESD workstation. Generally this resistor is integrated in the cord.

Short for ElectroStatic Discharge, ESD is one of the few things an individual can do to damage or destroy his or her computer or hardware components. Much like the shock you receive when rubbing your feet on the carpet and touching something metal, ESD can occur when working in your computer and will cause components you touch to no longer work properly. ESD can occur without the user feeling a shock and will only occur while working on the inside of the computer or handling an expansion card.

Preventing ESD and grounding yourself

The best method of preventing ESD is to use an ESD wrist strap, grounding mat, or grounding workbench. However, because most users do not have access to such items, we have included the below steps to help reduce the chances of ESD as much as possible.

Tip: If you plan on working on the inside of the computer more than once we highly recommend purchasing and using an anti-static wristband.

Zero Potential - Most importantly, make sure you and the computer are at Zero Potential by continuously touching an un-painted metal surface of the chassis or the computer power supply case.

Standing - It is also very important that you are standing at all times when working on the computer. Setting on a chair can generate more electrostatic.
Cords - Make sure everything is removed from the back of the computer (power cord, mouse, keyboard, etc).

Clothes - Make sure not to wear any clothing that conducts a lot of Electrical Charge, such as a wool sweater.

Weather - Electrical storms can increase the ESD risk; if it can wait, try not to work on a computer during an electrical storm.

Accessories - To help reduce ESD and help prevent other problem, it is also a good idea to remove all jewelry.

Surface - You should be standing on a hard surface, and the computer or other electrostatic sensitive device should be on a table.

SESSION 3 Explain environmental protection measures and procedures of a computer working environment.

Learning Outcomes

- The explanation covers handling of computer components. - The explanation covers handling of computer peripheral components.

Explain environmental protection measures and procedures of a computer working environment.

Working with computers

Risk assessment

The regulations require employers to carry out a risk assessment of users' workstations, which should consider the entire workstation, including equipment and furniture, as well as the work environment, eg lighting, temperature and leg room. The tasks that are being performed at the work station should be considered as should any special needs of individual staff.

Display screen equipment (DSE) risk assessments should also consider those factors that may contribute to repetitive strain injuries such as:

- sitting in the same position for a long period

- awkward positioning of the wrist and hand in relation to the keyboard

- high workload for a prolonged period of time

- excessive use of the mouse.

Checklist for workstations

The DSE Regulations detail the minimum standards for workstations, which are summarised below.

The display screen

This should:

- display well-defined characters of adequate size and spacing

- have a stable image

- have easily adjustable brightness and contrast

- tilt and swivel easily to suit the user

- be free from glare and reflections

- use a separate base for the screen, or an adjustable table.

The keyboard

This should:

- be tiltable and separate from the screen to allow the user to adopt a comfortable working position

- have a space in front to provide support for the hands or arms of the user

- have a matt surface

- have clearly legible symbols on the keys.

The work surface

The work surface should:

- provide adequate space for the user

- have a low reflective surface

- be of adequate size to allow the screen, keyboard, etc to be flexibly arranged

- have a stable, adjustment document holder, which should be at the same level as the screen and at the same viewing distance.

The work chair

This should have a seat that is adjustable in height, with a seat back adjustable in height and tilt. A footrest should be available.

The workstation/environment

The workstation must do the following:

- provide sufficient space for the user or the operator to alter position comfortably

- lighting must be adequate with suitable contrast between the screen and background

- glare and reflections on the screen should be avoided

- windows should be fitted with adjustable coverings to alter the daylight level.

When a workstation is shared by more than one person, it should be assessed in respect of each person.

The explanation covers handling of computer components.

Handling Computer Components

The internal components that make a computer do all those intricate and almost instantaneous processes are delicate pieces of electronics, and must be handled with care to prevent damage. Everything from the very dust in the air to the oil on your fingers can cause part failure. When handling computer parts, such as when installing or uninstalling internal components, a few key steps help to prevent any unintentional damage to the parts and keep you safe.

Clean and Clear

- Before working on your computer or handling any new internal components, wash and dry your hands thoroughly to remove any oil or other substances. If transferred to the computer's sensitive electronic parts, this residue can cause malfunction or even failure. Keep food and drink away from computers to prevent crumbs and spills, and clear a spot on your desk or floor to work without constriction. If working on your floor, vacuum first to remove any dirt, crumbs or lint from working their way into your computer.

Disconnect

- You can still receive a shock when working inside the case, even if your computer is completely powered down. Unplug the computer from the wall before installing or uninstalling any components within your case. Remove any cables that connect the tower to other accessories, such as the monitor, printer or external modem. Freeing your computer tower from everything connected to it will allow you freer access to the inside and greater range of motion. Remove the battery from a laptop and let it cool down completely before opening the case.

Static Free

- A static shock, such as the kind you feel when touching a door knob in the winter, can spell death for a computer's internal electronics. When handling computer components, always dissipate any built up static you may have by grounding yourself regularly. You can do this by touching any piece of metal, such as the metal casing of the computer tower or a door knob. Do not touch any of the electronic boards or other internal parts until you have successfully released this static electricity.

Careful Treatment

- Computer components are composed of complicated concoctions of complex circuitry. As such, these pieces require a bit of gentle handling to prevent knocks and damage. Always handle circuit boards such as motherboards and graphics cards by their edges, being careful not to touch the transistors and connecting pins. Hard drives contain moving parts inside, and can be damaged by rough handling or drops.

Proper Storage

- When storing your computer components, remember the three things that are most damaging to computers -- heat, dust and moisture. Store your computer components in a sturdy, well cushioned box away from heating sources and moist areas. Direct sunlight can cause the temperature inside a closed box to rise quickly, so find a dark, cool and dry area for your storage needs to keep your computer's electronic parts safe and in good condition.

Storage and transporting components

Correct storage and packaging when transporting your components, helps to ensure that your components remain in perfect working order. Water, damp, heat, ESD etc can contribute to faulty components so it’s best to avoid the disappointment altogether by taking necessary precautions in the first place.

Personally, I always keep boxes for all the components I buy. Firstly, it’s ideal for long term storage.

Secondly, if you decide to sell. Selling items in their original packaging not only makes a good impression on the buyer but also saves you the time and trouble finding alternatives which may be  unsuitable. Sending items in the packaging they were designed for is ideal for transportation.

Thirdly, returning faulty items to the retailer or manufacturer, some of which are quite strict about their packaging polices.

If you’re upgrading frequently, then storing your boxes for future use is a great way of eliminating damage when they are need and gives you some piece of mind too.

It’s also appreciated that this may not be convenient, so maybe ask someone who can store them for you. If you don't have the original storage boxes, then use what ever suitable packaging you can get.

Use anti-static bags wherever possible. You can get these cheap as chips from any high street electrical stores. It might even be worth asking if they have any spare.  Do place your components directly in to bubble wrap or any other similar material. Despite what people think, bubble wrap offers no ESD precaution.

Hard disks or any other magnetic sensitive material etc should be kept away from magnetic sources, and high voltage devices. If you are posting your components, make sure the package is clearly labelled “Static Sensitive” or words to that effect.

Make sure your packaging is secure and watertight.

Proper Handling of Computer Components

Perhaps of one the safety issue in PC assembly is the static electricity being discharged by the technican. It is no secret to us all the we generate static electricity by means of rubbing (either intentional or unitentional) surfaces and objects. Though this electricity, is not at high as a high - voltage wire, this electricity is what an electronic component needs to be damage. And so, as a PC technician as well, i show you how to handle computer components, the right way.

The figure above shows how we handle our expansion cards (and the like) properly. At least, if ever discharge static electricity, it does not go directly to the electronic component (ICs, resistors, ad capacitors)that is inside the expansion card.

The next figure shows improper handling of our expansion cards. If handled this way for a long time, it is no wonder that the performance of our components deteriorate. And then eventually will damage our component.

The explanation covers handling of computer peripheral components.

Computer Peripherals

Within the context of personal computing, the term "peripheral" refers to devices that connect to computers through external ports, such as printers, audio speakers, web cameras, routers, modems, card readers and exterior hard drives. Proper maintenance of computer peripherals varies widely by type and should be performed as recommended by the manufacturer. This article provides information on the most frequently suggested and universally applicable tasks and practices recommended for maintaining peripheral devices.

A peripheral is a device that is connected to a host computer, but not part of it. It expands the host's capabilities but does not form part of the core computer architecture. It is often, but not always, partially or completely dependent on the host.

There are three different types of peripherals:

- Input, used to interact with, or send data to the computer (mouse, keyboards, etc.)

- Output, which provides output to the user from the computer (monitors, printers, etc.)

- Storage, which stores data processed by the computer (hard drives, flash drives, etc.)

What are some examples of computer peripheral devices?

Answer: A computer peripheral, or peripheral device, is an external object that provides input and output for the computer. Some common input devices include:

- keyboard

- mouse

- touch screen

- pen tablet

- joystick

- MIDI keyboard

- scanner

- digital camera

- video camera

- microphone

Some common output devices include:

- monitor

- projector

- TV screen

- printer

- plotter

- speakers

There are also devices that function as both input and output devices, such as:

- external hard drives

- media card readers

- digital camcorders

- digital mixers

- MIDI equipment

# Storage and handling

## Manufacturer’s requirements

When handling computer equipment, it is advisable to follow the manufacturer’s guidelines on handling and storage. The most obvious place to find that information would be the User Guides/Manuals that accompany the product.

While some documentation can be difficult to find, in a cupboard full of manuals, it is also common to have no documentation for the equipment in printed form. These days, many of the manuals and manufacturer guidelines are in electronic form supplied on floppy disk or CD-ROMs.

One of the best avenues, to locate the current information, would be the Internet. If in doubt, go to the manufacturer’s website. While some manufacturer’s websites can be difficult to locate, there are many Internet directories that can be used to find them. For example, The Computer Information Centre at: www.compinfo-center.com/cmanuf.htm. Apart from documentation, the Internet provides user groups that discuss all manner of issues, where some will raise a question and others will provide the answers.

Your supplier is the other main avenue for valid information. A good supplier will already be aware of specific issues that relate to your product purchases. If your supplier is reluctant to provide relevant product information, find a new supplier.

Storing equipment

You will find that manufacturers will almost invariably require that equipment should be stored in the same packaging in which it was delivered. While this is valid, in principle, often it can be impractical. Empty packaging can consume significant storage space, which may seem not justifiable on a cost basis. However, if you do not have on-site support then to return equipment to the supplier, you will need enough to cover the basics. For example, if you have five printers from one manufacturer, you may choose to keep the packaging of one printer.

Just like locating equipment, when storing equipment you must consider the factors of temperature, humidity, dust etc. Although if equipment is not in use then such factors as temperature are less of an issue than if the equipment were in service ie in current use.

If unused or stored equipment is packaged similar to its original state, this will usually suffice. Any partially used consumables like ink or toner cartridges should not be stored, but disposed in the manner prescribed by the manufacturer. While this may seem wasteful, after a short time it is unlikely that the consumables will be in a useable state.